Traffic Management System – Test Cases

Functional Requirement Test Cases

| Test Case ID | Name of Module | Test case description | Pre-conditions | Test Steps | Test data | Expected Results | Actual Result | Test Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UT_01 | Simulation Control | To test whether the user can start the traffic simulation | Application is open and simulation is in stopped/reset state | - Open the application. - Click Start. - Observe the simulation. | Simulation state: Stopped | Simulation starts successfully and vehicles/signals begin operating. |  |  |
| UT_02 | Simulation Control | To test pause, resume, stop and reset operations | Simulation is running | - Click Pause. - Observe simulation. - Click Resume. - Click Stop. - Click Reset. | Running simulation | Pause stops simulation progression, Resume continues it, Stop ends it, and Reset returns the simulation to its initial state. |  |  |
| UT_03 | Vehicle Management | To test whether vehicles are generated dynamically | Simulation is running | - Start the simulation. - Observe vehicle generation over time. - Continue the simulation for several cycles. | Traffic density: Medium | Vehicles are generated/accepted dynamically during simulation without requiring manual creation of every vehicle. |  |  |
| IT_04 | Vehicle Management | To test whether incoming vehicles are assigned to the appropriate approach/lane | Simulation is running and multiple approaches are available | - Generate/accept an incoming vehicle. - Observe its assigned approach/lane. - Repeat for vehicles entering from different directions. | Vehicles entering from North, South, East and West approaches | Each vehicle is assigned to the correct corresponding approach/lane. |  |  |
| UT_05 | Vehicle Detection | To test detection and classification of normal vehicles | Simulation is running | - Introduce a normal vehicle. - Observe vehicle classification. | Vehicle type: Normal | Vehicle is detected and classified as a normal vehicle. |  |  |
| UT_06 | Vehicle Detection | To test detection and classification of emergency vehicles | Simulation is running | - Introduce an emergency vehicle. - Observe vehicle classification and status. | Vehicle type: Emergency | Vehicle is detected and classified as an emergency vehicle and marked for priority handling. |  |  |
| UT_07 | Queue Management | To test whether a separate queue is maintained for each approach/lane | Simulation is running | - Add vehicles to two different approaches. - Observe the queues. | North: 3 vehicles East: 2 vehicles | Separate queues are maintained for the North and East approaches, with vehicles appearing in their respective queues. |  |  |
| UT_08 | Queue Management | To test FIFO processing of normal vehicles | Vehicles are waiting in the same approach | - Add three normal vehicles sequentially. - Record their arrival order. - Allow the signal to turn green. - Observe processing order. | Vehicle order: V1 → V2 → V3 | Normal vehicles are processed in their arrival order: V1 → V2 → V3. |  |  |
| UT_09 | Traffic Demand | To test traffic demand calculation using queue length | Simulation is running | - Add vehicles to an approach. - Observe the queue length and calculated demand. - Add additional vehicles. - Observe demand again. | Initial queue: 2 vehicles Final queue: 6 vehicles | Traffic demand increases when the number of waiting vehicles increases. |  |  |
| IT_10 | Traffic Demand | To test whether waiting time contributes to traffic demand | Vehicles are waiting at an approach | - Place vehicles in an approach queue. - Allow them to wait for a period. - Observe traffic demand. | Waiting time: increasing over time | Traffic demand is updated as vehicle waiting time increases. |  |  |
| IT_11 | Traffic Signal Control | To test whether the next signal phase is selected based on traffic demand | Simulation is running | - Create high traffic demand on one approach. - Create lower demand on other approaches. - Allow the current phase to complete. - Observe the next phase. | North: 8 waiting vehicles East: 2 waiting vehicles | The next signal phase is selected dynamically based on the current traffic demand rather than a completely fixed cycle. |  |  |
| ST_12 | Traffic Signal Control | To test signal phase selection when traffic demand changes | Simulation is running | - Create high demand on Approach A. - Observe the selected phase. - Change demand so Approach B has higher demand. - Observe the subsequent signal decision. | Phase 1: A has higher demand Phase 2: B has higher demand | Signal decisions change according to the updated traffic demand. |  |  |
| UT_13 | Green Time Control | To test whether green-light duration increases with higher traffic demand | Simulation is running and minimum/maximu m green times are configured | - Create low traffic demand. - Record green duration. - Create high traffic demand. - Record green duration. | Low demand: 2 vehicles High demand: 10 vehicles | Green-light duration is adjusted according to traffic demand, with higher demand receiving an appropriately longer duration. |  |  |
| UT_14 | Green Time Control | To test whether green duration remains within configured limits | Simulation is running | - Configure minimum and maximum green times. - Create very low demand. - Create very high demand. - Observe green durations. | Minimum: 10 sec Maximum: 60 sec | Green-light duration never falls below 10 seconds or exceeds 60 seconds. |  |  |
| IT_15 | Traffic Flow | To test whether vehicles move when their approach has a green signal | Vehicles are waiting at an approach | - Place vehicles in the approach queue. - Set the approach signal to green. - Observe the vehicles. | Signal: Green Waiting vehicles: 3 | Vehicles move through the intersection when their approach receives a green signal. |  |  |
| IT_16 | Traffic Flow_1 | To test whether vehicles remain waiting during a red signal | Vehicles are waiting at an approach | - Place vehicles in the approach queue. - Set the approach signal to red. - Observe the vehicles. | Signal: Red Waiting vehicles: 3 | Vehicles remain waiting and do not cross the intersection while the signal is red. |  |  |
| IT_17 | Emergency Vehicle Handling | To test whether an emergency vehicle receives priority | Simulation is running with normal traffic | - Add normal vehicles to an approach. - Introduce an emergency vehicle. - Observe signal operation and vehicle movement. | Normal vehicles: 5 Emergency vehicles: 1 | The emergency vehicle is given priority and signal operation is adjusted to allow safe passage. |  |  |

| ST_18 | Emergency Vehicle Handling | To test whether emergency priority maintains intersection safety | Conflicting traffic is present | - Introduce an emergency vehicle. - Observe the signals for conflicting approaches. - Observe emergency vehicle movement through the intersection. | Emergency vehicle: 1 Conflicting approach: vehicles waiting | Emergency priority is provided without giving conflicting approaches green simultaneously or allowing an unsafe movement. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IT_19 | Adaptive Signal Controller | To test whether traffic conditions are re-evaluated after each signal phase | Simulation is running | - Run one signal phase. - Change the number of waiting vehicles. - Allow the phase to complete. - Observe the next signal decision. | Initial queue: 2 vehicles Updated queue: 8 vehicles | The system re-evaluates traffic conditions after the phase and adjusts the next phase based on the updated demand. |  |  |
| ST_20 | Adaptive Signal Controller | To test continuous adaptation to changing traffic conditions | Simulation is running | - Run the simulation through multiple signal phases. - Change traffic demand between phases. - Observe signal decisions after each phase. | Demand changes between phases | The signal controller continuously updates its decisions according to changing traffic conditions. |  |  |
| UT_21 | Congestion Detection | To test congestion detection when queue length exceeds the threshold | Simulation is running and congestion threshold is configured | - Configure congestion threshold. - Add vehicles until queue length exceeds the threshold. - Observe congestion status. | Congestion threshold: 10 vehicles Queue length: 11 vehicles | The system detects congestion and indicates the congestion status. |  |  |
| UT_22 | Congestion Detection | To test that congestion is not indicated below the threshold | Simulation is running and congestion threshold is configured | - Configure congestion threshold. - Add vehicles below the threshold. - Observe congestion status. | Congestion threshold: 10 vehicles Queue length: 5 vehicles | Congestion is not indicated while traffic demand/queue length remains below the defined threshold. |  |  |
| IT_23 | Traffic Statistics | To test whether traffic statistics are calculated and displayed | Simulation has processed vehicles | - Run the simulation. - Allow vehicles to be processed. - Open/view the statistics section. | Processed vehicles: 10 Waiting vehicles: 5 | Statistics including vehicles processed, average waiting time, maximum waiting time and queue length are calculated and displayed. |  |  |
| ST_24 | Traffic Statistics | To test whether congestion events are included in statistics | Simulation is running and congestion detection is enabled | - Run the simulation with traffic exceeding the congestion threshold. - Allow congestion to occur. - View the statistics. | Congestion events: 2 | The statistics display the number of congestion events along with the other traffic statistics. |  |  |